Attachment No. 1: Model offer form

Name and address of the Contractor:……………………………………………………

Contact person..…………………………………Phone number………………………………… e-mail: ………………………………………………….

Subject of the contract: A set for non-invasive recording of cell electrical activity and analysis of data from multi-electrode matrices

|  | DESCRIPTION and scoring of parameters and requirements | Compliance with the requirement (YES/NO) | NOTES |
| --- | --- | --- | --- |
| 1. | A set for non-invasive recording of cell electrical activity and analysis of data from multi-electrode matrices - A workstation for recording and analyzing data from one 60 electrode microarray with features: - Recording and analysis of data from at least 60-electrode microarray and at least 8 additional analogue channels - There should be a stimulator in the workstation, allowing selective stimulation of selected electrodes through the built-in software - The set should include an integrated amplifier with a signal filtering system and at least a 2-channel stimulus pulse generator - Integrated workstation heating system - The station is capable of operating in an incubator with controlled levels of oxygen and carbon dioxide - Possibility of electrical recording/stimulation with simultaneous imaging under the inverse microscope in transient light and wide field fluorescence - Possibility to extend the set with additional workstations used in in vitro /ex vivo experiments - Possibility of expanding the system with similar units (workstations) - Option to extend the system with a perfusion system - The set should include software for data recording and analysis with the possibility of free updates - Data rate (USB) not less than 50 kHz/channel - Free technical assistance for at least 2 years - Multi-electrode microarrays compatible with a reusable workstation and a simple and quick method of sterilisation (including autoclaving) - Features of multi-electrode microarrays: - the option of installing dedicated membranes, permeable to oxygen and carbon dioxide and not permeable to water - reducing the risk of liquid evaporation and biological infection during experiments - possibility of simultaneous high-resolution imaging in inverted microscopy and electrical stimulation - Transparent bottom for high-resolution inverse microscope imaging - Analysis of nerve cell electrical activity documented by publications available in PubMed database (at least 30 publications from 2015) - Life time electronic contact with experts with laboratory experience in the use of equipment | (YES/NO) (YES/NO) (YES/NO) (YES/NO) (YES/NO) (YES/NO) (YES/NO) (YES/NO) (YES/NO) (YES/NO) (YES/NO) (YES/NO) (YES/NO) (YES/NO) YES/NO) (YES/NO) (YES/NO) (YES/NO) |  |

Net…………………………………………………………… price ., .……….%VAT………………… ..

Gross…………………………………………………………………………… price

Order completion date .…………weeks from the date of signing the contract (max. 9 weeks)

Warranty period:…………………………………………………………………(min. 2 years)

(0 points – 2 years, 10 points - 3 years, 20 points – 4 years)

Publicly documented results obtained from 2015 using the set described in the documentation  - Number of publications……………………………………… (30  publications- 0 points,  31-100 publications- 10 points, over 100 publications – 20 points)

I have read the model agreement attached as Appendix No. 2 to the inquiry and I do not raise any objections to it

……………………………………………                                                               …………………......…………………………          		      Place, date                                                                       	Signature and stamp of the Contractor

Pasteur 3, 02-093 Warsaw, tel: (48-22) 589 23 82; e-mail: k.kalita@nencki.edu.pl; http://www.nencki.gov.pl_