Warsaw, 09.03.2020

Request for quotation No. 33/2020

for the delivery

Single-channel high power electric stimulator

to the Marceli Nencki Institute of Experimental Biology

Polish Academy of Sciences

Ordering party: Institute of Experimental Biology named after M. Nencki, Polish Academy of Sciences, with its registered office at 3 Pasteura Street, Warsaw (02-093), NIP: 525-000-92-69, REGON: 000325825

Contact person for the contract: Piotr Michaluk

e-mail: p.michaluk@nencki.edu.pl

Deadline for submission of tenders: no later than 13.03.2020, to 12:00 p.m.

I. Description of the subject matter of the contract:

The subject of the contract is:

- Single-channel high power electric stimulator with the following features:

- Possibility to switch between constant current and constant voltage modes

- Isolated high power output signal

- No batteries required for output operation

- Maximum voltage: 200 V

- Maximum current: 100 mA

- Minimum pulse time: 1 microsecond

- Maximum pulse time: 25 hours

- Can be triggered with a TTL signal sent from external devices and has a built-in BNC socket for this purpose

- Can be controlled via the device's control panel and via software installed in Windows 10 environment

- Output waveform types: monophasic, biphasic, ramps, user generated custom waveforms

- For use in a 220 V / 50 Hz power system

- Warranty: min. 12 months

II Criteria for the evaluation of tenders

In the selection process, the Contracting Authority shall be guided by the price criterion while fulfilling the conditions for participation in the procedure.

III Description of the Preparation of the Offer and its Evaluation:

- The offer should be prepared on model No. 1 attached to the Inquiry.

- The offer should include information about the total net and gross value of the order: Before signing the contract, the Economic Operator, whose bid will be selected, will deliver the following scans: certificates of entry into the business register, REGON certificate and NIP certificate.

- Tenders must be sent electronically in the form of a scanned original tender by e-mail to the following address: p.michaluk@nencki.edu.pl.

- Please mark the offer in the message title: Single-channel high power electric stimulator.

- Only offers containing all the elements listed above will be evaluated.

IV Additional information:

- In order to complete the order, a contract will be signed with the selected Contractor.

- The maximum completion date for each order under the contract is max. to 6 weeks (the declared delivery date is indicated by the Contractor in the offer).

- The Contracting Authority reserves the right to negotiate the terms of the contract with the best contractors.

- The Contracting Authority reserves the right not to select any of the Economic Operators.

- The choice of the Contractor will be announced on the website. The contracting authority shall do so without delay after the end of the procedure.

Pasteura 3, 02-093 Warszawa, tel.: (48-22) 589 21 80;, e-mail: w.boguta@nencki.edu.pl; http://www.nencki.gov.pl_